GUIDANCE CHECKLIST ON CONCENTRATED INSPECTION CAMPAIGN ON BALLAST WATER MANAGEMENT

| No. | Item | Guidance and checks | Verified by Master |
| --- | --- | --- | --- |
| 1. | Is a valid International Ballast Water Management Certificate (IBWMC) on board? | - Check ship’s particulars are correct. - Check same total Ballast Water Capacity compared with BWMP and Stability book. - Only D-2 is ticked off. - Correct issue date and validity date of the IBWMC. - Certificate endorsed for relevant periodical surveys. |  |
| 2. | Is the approved Ballast Water Management Plan (BWMP) on board? | - Check approval stamp by Class on cover page - Change of Class is endorsed by new Class surveyor. - Change of ship’s name and particulars (if any) endorsed by Class surveyor. |  |
| 3. | Is the BWMP up to date to reflect the applicable requirements to manage Ballast Water as required by the Convention? | - Check correct listing of BW tanks in BWMP. - D-1 exchange method is only indicated as contingency measure. - New addendum to BWMP recommended as applicable (Class approval for Singapore flag vessel) for treated sewage / grey water in BW tanks and challenging water quality (CWQ) in port - Chief Officer and 2/E are familiar with the action to be taken in case of CWQ - CWQ incident is recorded in BWRB (refer the examples provided in the initial pages of the BWRB). - New Form of BW record book and guidance BWM.2/Circ.80/Rev.1 (Circular to be kept in BWMP, same uploaded in Share Point for easy reference) - New ballast water reporting Form is available as per BWM.2/Circ.80/Rev.1 is in BWMP. |  |
| 4. | Are officers and crew familiar with their duties in the implementation of the BWMP? | - Whether training has been carried to crew on board as per BWMP and training record available on board. - Confirm Engineers are familiar with the operation of BWTS, alarms and maintenance. - Confirm crew aware of safety aspects associated with the particular system (BWTS) and procedure on board. - Duty Officer should be familiar with following: - Methods used for managing water and sediments - Onboard equipment and structure - Key content in BWMP - BWTS fault analysis and troubleshooting - Contingency measures (including CWQ if any) |  |
| 5. | Is the Ballast Water Management System (BWMS) approved by the Administration/Organization, as appropriate? | - Confirm BWTS Type approval certificate (TAC) is available on board. - Confirm TAC issued by a Flag or by an RO on behalf of a Flag. - The TAC must have been valid at the time of BWMS installation. (the TAC can have expired validity, but it should be valid at the time of BWTS installation) - Calibration certificates of relevant sensor/meters are available and are valid |  |
| 6. | Is the BWMS operational? | - PSC can check operation of BWTS, ensure all Engineers are able to operate it and explain various alarms. - Confirm that regular maintenance has been carried out as per maker’s manual? (Check MESPAS for PMS). - In case of BWTS malfunction, inform office immediately for getting dispensation letter from flag state and for arranging the repair. - Check following common deficiencies as applicable - No visible deficiencies on valves, pipes, sensors, control panels etc. - Any abnormal alarms indicated on the control panel - Log information in the BWTS not in line with the record in BWRB - BWTS faults are not correctly recorded in the BWRB - Faults are not reported to office/flag state/port state - Calibration certificates of measuring cells are invalid - Emergency measures are not implemented in compliance with approved BWMP. |  |
| 7. | Was the Ballast Water managed according to the BWMP? | - Ensure BWTS is used for each ballasting/de-ballasting as per IBWMC D-2. - Holding time of ballast water in BW tanks to be followed if required by the TAC. - Bypass of the BWTS is nor allowed and shall give an alarm. - Bypass of BWTS only under special conditions (e.g. CWQ in port etc) after informing office and necessary approval from Flag and Port states. |  |
| 8. | Is the Ballast Water Record Book (BWRB) properly filled including exemptions if granted? | - Refer 1.3 How to record operations in the company Ballast Water Record Book. - Refer Appendix 1 Examples provided in the company Ballast Water Record Book for making correct entries. - Ensure correct Code letter and item number are in use. - Each operation concerning Ballast Water is recorded without any delay. - Each entry signed by the officer in charge of operation and each completed page signed by the Master. |  |
| 9. | Is the crew managing Ballast Water sediments in accordance with the BWMP? | - Familiarize with the management of sediments provided in the BWMP. - Disposal of sediments shall be done according to the approved BWMP (removal to port reception facility or disposal by tank flushing at sea 200nm from coast at 200nm water depth). - Removal of sediments + flushing of tanks is to be entered in the BWRB using Code letter G. |  |
| 10. | If an exemption has been granted, are the conditions of exemption implemented? | Not applicable |  |

Also note following for your preparation, inform us if any document is not available.

Ensure officers are familiar with BWMS tests.

BWMS commissioning test report is required BWTS installed after 8 September 2019.