| S.N | Item | YES | NO | N/A |
| --- | --- | --- | --- | --- |
| 1 | Fuel oil quick closing valves not bolted/tampered with (Note: The majority of serious engine room fires are fueled by oil. In the event of fire it is essential that the primary source of the fuel is rapidly isolated either by closing the valve locally or remotely) |  |  |  |
| 2 | Ensure all hot surfaces that are potential ignition sources are properly covered with lagging material (located above or near units of temperature above 220C including boilers, steam pipes, exhaust manifolds, silencers as per SOLAS II-2 regulation 4 - 2.2.6) - Laggings should not be missing - Lagging and insulation should be in good condition, tight and free from oil. The oil soaked lagging is to be renewed. - Ensure the proper maintenance of valve glands and pipe joints to prevent the leakage oil to lagging. |  |  |  |
| 3 | Fuel and lubricating systems and piping inspected closely for leakages from source tanks to system end points. Check for system vulnerabilities like cracks , loose or missing pipe clamps and securing devices, wear or chaffing due to vibration impacting hoses, nuts and bolts and piping or tubing which may be insufficiently secured |  |  |  |
| 4 | Ensure the joints of fuel oil supply pipes and fittings e.g. pressure gauges, thermometers /thermometer pockets , gaskets are in good order and ensure that there is no loosening or crack due to excessive vibration |  |  |  |
| 5 | Ensure there is no non-metallic flexible hoses in systems carrying flammable liquids particularly around engine areas where failures leading to leakage or spray may find hot spots capable of igniting the fluids. Consult with Ship Manager if modifications are needed to minimize the risk of fuel spray fires. |  |  |  |
| 6 | Ensure fuel supply pumps are properly examined, shaft sealing checked for leakages and bearings when fitted checked for overheating and indications of wear. |  |  |  |
| 7 | All heat sources particularly with respect to engine exhausts examined for hot spots. Areas where exhaust piping may exit the space and proceed through other spaces are thoroughly examined. |  |  |  |
| 8 | Ensure precautions are taken to prevent any oil that may escape under pressure from any pump, filter or heater from coming into contact with heated surfaces. |  |  |  |
| 9 | Ensure that spray shielding is kept in place where used and consider adding such shielding around gasketed flanges and other areas if helpful. For ALL LO & FO pipes with over 1.8 bar pressure - Flanges are to have anti splash tape fitted if they are not protected by some other means (steel defector plate) |  |  |  |
| 10 | Ensure fire detectors are not covered or disabled and are tested as per PMS |  |  |  |
| 11 | Ensure that no pressurized pipe or equipment including valves , bolts , flanges , fuel filters/strainers is opened until they are totally isolated and complete release of internal pressure is confirmed |  |  |  |
| 12 | Ensure parts around fuel oil purifier, fuel injection pump and high-pressure pipe around Main Engine and Auxiliary Diesel Engine are checked carefully during routine rounds |  |  |  |
| 13 | Ensure that the oil leakage alarm chamber is checked at regular intervals and is in good working order |  |  |  |
| 14 | Check maker manual and verify if ME indicator cocks require insulation and if so are in good order. |  |  |  |

Verified by CEO:                                                      DATE:

Examples of areas of high potential for engine room fires (i.e the areas where equipment, which contain pressurized oil and hot surfaces or naked fire coexist in vicinity to each other).

- Diesel Engine (Main Engine, Auxiliary Engine)

- Fuel oil supply pipe and fittings (union joint, pipe joint for gauge, drain valve, etc.)

- Turbo charger and exhaust gas pipe, manifold

- Lubricating oil supply pipe (including hydraulic oil for exhaust valve)

- Fuel oil strainer

- Lubricating oil strainer

- Supply and return pipe of fuel valve cooling oil and other accessory for generator engine

- Auxiliary Boiler and incinerator

- Burner fuel supply pipe and other accessory

- Exhaust pipe

- Fuel Oil Heater

- Fuel Oil Purifier and Decanter

- Lubricating Oil Heater

Sample locations of where engine room fires have occurred on some ships